Title

HW-SW SystemC Co-Simulation SoC Validation Platform

Interconnect Methodology Summary

Contract No. 4200022968

by

TU Braunschweig, Germany

September 10, 2012

Document No.: IDA-SCSV-IMS-001

Table of Contents

1	Introduction	4

1.1	Purpose and Scope	4

1.2	Revisions	4

2	AMBA interconnect	5

2.1	AHB Modeling	5

2.1.1	AHB Payload	5

2.1.2	AHB Protocol mapping	6

2.1.3	Not supported or partially supported features	9

2.1.4	Creating/Binding AHB sockets	9

2.2	APB Modeling	11

2.2.1	APB Payload	11

2.2.2	APB Protocol mapping	11

2.2.3	Creating/Binding APB sockets	12

2.3	AXI Modeling	13

2.3.1	AXI Payload	13

2.3.2	AXI Protocol mapping	14

2.3.3	Not supported or partially supported features	17

2.3.4	Creating/Binding AXI sockets	17

Table of Tables

Table 1 - Revisions of this document	4

Table 2 - AHB TLM payload mapping	6

Table 3 - APB TLM payload mapping	11

Table 4 - AXI TLM payload mapping	14

Table of Figures

Figure 1 - AHB write transfer (single)	7

Figure 2 - AHB read burst	8

Figure 3 - AHB write burst	9

Figure 4 - APB blocking transport delay	12

Figure 5 - AXI read burst	15

Figure 6 - AXI overlapping read bursts	16

Figure 7 - AXI write burst	17

# Introduction

## Purpose and Scope

This document is the Interconnect Methodology Summary of the SystemC Co-Simulation SoC Validation Platform (SoCRocket). It describes the IP interconnect infrastructure of the library, particularly the modeling of the AMBA protocol at loosely timed and approximately timed levels of abstraction.

## Revisions

The following table will be updated during the course of the project.

| Version | Date | Description |
| --- | --- | --- |
| 0.1 | 09/09/12 | Initial version of document |

Table 1 - Revisions of this document

# AMBA interconnect

Almost all components of the library are equipped with an AMBA interface. At TLM level these interfaces are represented by sockets, which are customized for interchanging payload of a certain format. The AMBA sockets used for modeling the SoCRocket communication interfaces are supplied by a TLM AMBA Modeling Kit, which has been developed under the roof of GreenSoCs. The kit is distributed by Carbon Design Systems Inc. and can be freely downloaded and used. For instructions please see the SoCRocket User Manual.

## AHB Modeling

The AHB protocol is modeled at loosely timed and approximately timed level of abstraction. Focus is on high-level, functionally accurate transaction modeling. Low-level signals such as, for example, channel handshakes are not important at this level. Modeling the AHB protocol at transaction level requires a customized payload (2.1.1) and a mapping of TLM phases to protocol synchronization points (2.1.2).

### AHB Payload

For modeling AHB bus communication transfer information is either mapped to fields of the TLM generic payload or to a sub-set of the payload extensions provided by the Carbon/GreenSoCs TLM AMBA Modeling Kit (Table 2).

| AHB Signal | Description | Mapping |
| --- | --- | --- |
| HMASTER | ID of the AHB master the initializes the transaction | amba_ext::amba_ID |
| HADDR | Target address of the transaction | tlm_generic_payload::address |
| HTRANS | Type of current transfer: nonseq, seq, busy | Burst implicitly identified by length of payload. tlm_generic_payload::data_length |
| HWRITE | Write or read transfer | tlm_generic_payload::tlm_command |
| HSIZE | Size of the transfer in bytes | tlm_generic_payload::data_length |
| HBURST | Type of burst (e.g. incremental, wrapping) | Burst implicitly identified by length of payload. tlm_generic_payload::data_length |
| HPROT | Protection against illegal transactions | cacheability: amba_ext::cacheable not supported: instr/data access, normal/privileged access, bufferable |
| HWDATA | Write data | tlm_generic_payload::data_ptr |
| HRDATA | Read data | tlm_generic_payload::data_ptr |
| HRESP | Transfer response | tlm_generic_payload::tlm_response |
| HLOCK | Indicates that master requires locked access to bus | amba_ext::lock |
| HSPLIT | Indicates split transfer | not supported |
| HREADY | Target ready to receive/send data | Low cycles are modeled by delaying BEGIN_RESP |
| HVALID | Initiator ready to receive/send data | Low cycles are modeled by delaying END_RESP |

Table 2 - AHB TLM payload mapping

### AHB Protocol mapping

#### LT Abstraction

At LT abstraction AHB transfers are modeled using plain TLM 2.0 blocking transfer calls (b_transport). The initiator starts the transaction at the beginning of the AHB address phase. The delay of the interconnect components and the target is aggregated in the transaction and returned to the initiator. The initiator is responsible for synchronization and may or may not decide to run ahead of time.

#### AT Abstraction

At AT abstraction AHB transfers are modeling using four timing points. The timing points relate to the phases to the TLM 2.0 standard protocol. However, in order to model timing in a more accurate way one additional phase transition was required. In following we will explain all relevant use cases in detail.

AHB write/read transfer (single)

Figure 1 shows the TLM phase assignment for a single-beat write transfer:

- Initiator sends BEGIN_REQ at beginning of AHB address phase

- Target sends END_REQ at the end of the AHB address phase (address sampled by slave)

- Target sends BEGIN_RESP at the beginning of the AHB data phase; corresponds to HREADY becoming high (slave ready - wait states over). The delay of BEGIN_RESP should also reflect the wait states produced by the target during the data phase (not just the once before accepting the first data item).

- Initiator sends END_RESP at the end of the AHB data phase. END_RESP must be delayed by the number of cycles VALID was low (transmission delayed by master).

Figure 1 - AHB write transfer (single)

AHB read burst

Figure 2 shows the phase protocol mapping for an AHB read burst:

- The initiator sends BEGIN_REQ at the beginning of the AHB address phase

- The target will send BEGIN_RESP at the beginning of the AHB data phase.
BEGIN_RESP is delayed by the number of wait states. This includes both: the wait states before delivering the first data item and the intermediate wait states inserted during transfer (target blocking).

- Because address phase and data phase overlap during an AHB burst transfer, END_REQ will usually be send after BEGIN_RESP. END_REQ marks the end of the AHB address phase. It relates to the time when the target samples the last address of the burst.

- The initiator sends END_RESP at the end of the AHB data phase. END_RESP will be delayed by the number of initiator stall cycles (VALID low).

Figure 2 - AHB read burst

AHB write burst

The synchronization points of AHB write bursts are similar to AHB read bursts. The full mapping is shown in Figure 3:

- The initiator sends BEGIN_REQ at the beginning of the AHB address phase.

- The target sends BEGIN_RESP at the time the first data item is sampled, delayed by the total number of wait states involved in the transactions (cycles HREADY is low). This includes both: the wait states before delivering the first data item and the intermediate wait states inserted during transfer (target blocking).

- Because address phase and data phase overlap during an AHB burst transfer, END_REQ will usually be send after BEGIN_RESP. END_REQ marks the end of the AHB address phase. It relates to the time when the target samples the last address of the burst.

- The initiator sends END_RESP at the end of the AHB data phase. END_RESP will be delayed by the number of initiator stall cycles (VALID low).

Figure 3 - AHB write burst

### Not supported or partially supported features

The model does not support early burst termination.

In case of a data split the slave is supposed to delay BEGIN_RESP. The arbiter (ahbctrl) will not use the additional delay to schedule another master.

### Creating/Binding AHB sockets

The easiest way to create custom components with AHB sockets is to inherit from one of the AHB modeling base classes: AHBMaster, AHBSlave. The procedure is explained in detail in the SoCRocket User Manual.

For manual instantiation include header amba.h from Carbon TLM AMBA modeling kit.

Master socket declaration (single):

amba::amba_master_socket<32> ahb;

Instantiation (module constructor):

ahb("ahb", amba::amba_AHB, ambaLayer*, false**);

*ambaLayer = amba::amba_AT or amba::amba_LT

** Only used for CT modeling

Slave socket declaration (single):

amba::amba_slave_socket<32> ahb;

Instantiation (module constructor):

ahb("ahb", amba::amba_AHB, ambaLayer*, false**);

*ambaLayer = amba::amba_AT or amba::amba_LT

** Only used for CT modeling

Multi-master socket declaration:

amba::amba_master_socket<32, 0*> ahbIN;

* Number of channels for this socket - 0 meaning no limit

Instantiation (module constructor):

ahbIN("ahbIN", amba::amba_AHB, ambaLayer*, false**)

*ambaLayer = amba::amba_AT or amba::amba_LT

** Only used for CT modeling

Multi-slave socket declaration:

amba::amba_slave_socket<32, 0*> ahbOUT;

* Number of channels for this socket - 0 meaning no limit

Instantiation (module constructor):

ahbOUT("ahbOUT", amba::amba_AHB, ambaLayer*, false**)

*ambaLayer = amba::amba_AT or amba::amba_LT

** Only used for CT modeling

Binding sockets:

Binding AHB sockets follows the rules for TLM2.0 socket binding.

initiator.ahb(target.ahb)

See ./platforms/leon3mp/sc_main.cpp for more examples.

## APB Modeling

The APB protocol is modeled at loosely timed level of abstraction. Focus is on high-level, functionally accurate transaction modeling. Low-level signals such as, for example, channel handshakes are not important at this level. Modeling the APB protocol at transaction level requires a customized payload (2.2.1) and adequate target delay estimation (2.2.2).

### APB Payload

For modeling APB bus communication transfer information is either mapped to fields of the TLM generic payload or to a sub-set of the payload extensions provided by the Carbon/GreenSoCs TLM AMBA Modeling Kit (Table 3).

| APB Signal | Description | Mapping |
| --- | --- | --- |
| PADDR | Target address of transfer | tlm_generic_payload::address |
| PSEL/PENABLE | Bridge selects slave (decoder signal) | not required |
| PWRITE | Read or write operation | tlm_generic_payload::tlm_command |
| PWDATA | Write data | tlm_generic_payload::data_ptr |
| PRDATA | Read data | tlm_generic_payload::data_ptr |
| PREADY | Target ready to deliver data (extension of data phase) | Implicitly modeled Target increments transactions delay in blocking transport. |

Table 3 - APB TLM payload mapping

### APB Protocol mapping

The APB protocol is intended for low-bandwidth communication with I/O components or memory mapped control registers. In contrast to AHB or AXI, APB is not pipelined and can therefore be sufficiently modeled using blocking communication.

Figure 4 shows a simple APB write transfer and its abstraction using a blocking transport call:

- The initiator calls b_transport at T2.

- The target will instantly return the aggregated component and setup delays (2 cycles).

- The initiator receives the delay a blocks the bus.

- The next transaction may not be issued before T4.

Figure 4 - APB blocking transport delay

### Creating/Binding APB sockets

The easiest way to create custom APB components is to instantiate a GreenReg socket. GreenReg sockets are extended AMBA sockets, which allow the specification of register interfaces.

Please include the amba.h header of the Carbon AMBA TLM Modeling Kit and the greenreg_ambasockets.h header in your design.

A detailed description on how to set up APB components and register interfaces is given in the SoCRocket library user manual.

Master socket declaration (single):

gs::reg::greenreg_socket<gs::amba::amba_slave<32> > apb;

Instantiation (module constructor):

apb("apb", 
		r,                                              	// Name of GreenReg register container
		APBDevice::get_base_addr_(), 		// APB base address
		APBDevice::get_size_(),					// APB size of address space
		amba::amba_APB,							// Bus protocol specifier
		amba::amba_LT,								// APB is always LT (blocking)
		false),												// Only used for CT modeling

## AXI Modeling

The AXI protocol is modeled at loosely timed and approximately timed level of abstraction. Focus is on high-level, functionally accurate transaction modeling. Low-level signals such as, for example, channel handshakes are not important at this level. Modeling the AXI protocol at transaction level requires a customized payload (2.3.2) and a mapping of TLM phases to protocol synchronization points (2.3.3).

Remark: The SoCRocket library currently contains no components with AXI interface. Nevertheless, in order to be prepared for future extensions, the build-in Carbon/GreenSoCs AMBA sockets support a set of features for AXI modeling. In this chapter we also propose a TLM phase mapping for approximating the AXI protocol.

### AXI Payload

For modeling AXI bus communication, transfer information is either mapped to fields of the TLM generic payload or to a sub-set of the payload extensions provided by the Carbon/GreenSoCs TLM AMBA Modeling Kit (Table 4).

| AXI Signal | Description | Mapping |
| --- | --- | --- |
| AWADDR | Target address of write operation | tlm_generic_payload:: address |
| ARADDR | Target address of read operation | tlm_generic_payload:: address |
| AWSIZE/ARSIZE | Size of transfer in bytes | tlm_generic_payload:: data_length |
| AWLEN/ARLEN | Number of transfers within a burst | Implicitly modeled using payload length (assuming constant bus width) tlm_generic_payload:: data_length |
| AWBURST/ARBURST | Type of burst | Implicitly modeled using payload length tlm_generic_payload:: data_length |
| ARCACHE/AWCACHE | Read/write cached | not required |
| ARPROT/AWPROT | Access protection mechanisms | not supported: normal/privileged access secure/non-secure, instruction/data |
| AWLOCK/ARLOCK | Initiator requires exclusive or locked access (for e.g. atomic operations) | ext::amba_exclusive ext::amba_lock |
| BRESP/RRESP | Write/read response | Modeled by TLM phase END_RESP |
| AWID, ARID, WID, RID, BID | Address IDs, write/read data IDs, response ID | ext::amba_ID |
| WDATA | Write data | tlm_generic_payload:: data_ptr |
| RDATA | Read data | tlm_generic_payload:: data_ptr |

Table 4 - AXI TLM payload mapping

### AXI Protocol mapping

#### LT Abstraction

At LT abstraction AXI transactions should be modeled using plain TLM 2.0 blocking transfer calls (b_transport). The initiator starts the transaction at the beginning of the AXI address phase. The delay of the interconnect components and the target is aggregated in the transaction and returned to the initiator. The initiator is responsible for synchronization and may or may not decide to run ahead of time.

#### AT Abstraction

At AT abstraction AXI transfers are modeling using four timing points. We propose a phase mapping for the individual transactions equal to AHB. The difference between AXI and AHB is rather related to the handling of transactions at initiator and target side then to the mapping of the TLM synchronization points. In contrast to AHB, AXI provides distinct channels for address/command, read data, write data, and response. Targets may accept multiple transactions from one or multiple targets in a row and reply to them out-of-order.

AXI read/write (single)

Equivalent to respective AHB transfers (see section 2.1.2.2)

AXI Read burst

Figure 5 shows a 4-beat AXI read burst. In contrast to AHB bursts the address bus only submits the starting address of the burst. Therefore, address and data phase do not overlap:

- The initiator sends BEGIN_REQ to start the transactions. This is the start of the AXI address phase.

- The target replies by sending END_REQ at the end of the AXI address phase (address sampling time)

- The target now sends BEGIN_RESP. The delay of BEGIN_RESP must include all wait states: the wait states before delivering the first data item and the intermediate wait states inserted during transfer (target blocking - READY low).

- The initiator sends END_RESP at the end of the AXI data phase. END_RESP will be delayed by the number of initiator stall cycles (VALID low).

Figure 5 - AXI read burst

Overlapping read bursts

Figure 6 shows two overlapping read bursts. As already mentioned, in contrast to AHB, only the first address of a burst is submitted to the target. Therefore, the address/command bus is instantly available for other transactions to start:

- T0 - Begin of address phase first transaction (blue). Initiator sends BEGIN_REQ.

- T2 - End of address phase first transaction (blue). Target replies with END_REQ.

- T2+x - Begin of address phase second transaction (red). Initiator sends another BEGIN_REQ

- T3 - Read data for first transaction (blue) is ready. Target sends BEGIN_RESP.

- T4 - Address of second transaction (red) sampled by target (end of address phase). Target sends second END_REQ.

- T9 - End of data phase first transaction (blue). Initiator sends END_RESP.

- T9+x - Data bus now free for second transaction (red). Target sends another BEGIN_RESP.

- T12 - End of data phase second transaction (red). Initiator sends END_RESP.

Figure 6 - AXI overlapping read bursts

AXI write burst

Figure 7 shows the proposed TLM phase mapping for an AXI write burst:

- The initiator starts the transaction by sending BEGIN_REQ (start of AXI address phase)

- The target replies with END_REQ at the time the address is being sampled (end of AXI address phase).

- The target sends BEGIN_RESP after all wait states have been consumed: wait states before accepting first data item, and intermediate wait states caused by target blocking (READY=low)

- The initiator sends END_RESP at the end of the AXI data phase. END_RESP must be delayed by the number of initiator blocking cycles (VALID=low cycles).

Figure 7 - AXI write burst

### Not supported or partially supported features

The proposed TLM-phase mapping for AXI does not allow transaction interleaving. This means: idle cycle on the data bus cannot be used to serve other pending transactions. This feature can only be modeled by introducing additional phases in the TLM protocol, which increases the implementation effort and slows down simulation. The impact on simulation accuracy strongly depends on the use case.

### Creating/Binding AXI sockets

For instantiating AXI sockets include the amba.h header from Carbon/GreenSocs TLM AMBA modeling kit in your design.

Master socket declaration (single):

amba::amba_master_socket<32> axi;

Instantiation (module constructor):

axi("axi", amba::amba_AXI, ambaLayer*, false**);

*ambaLayer = amba::amba_AT or amba::amba_LT

** Only used for CT modeling

Slave socket declaration (single):

amba::amba_slave_socket<32> axi;

Instantiation (module constructor):

axi("axi", amba::amba_AXI, ambaLayer*, false**);

*ambaLayer = amba::amba_AT or amba::amba_LT

** Only used for CT modeling

Multi-master socket declaration:

amba::amba_master_socket<32, 0*> axiIN;

* Number of channels for this socket - 0 meaning no limit

Instantiation (module constructor):

axiIN("axiIN", amba::amba_AXI, ambaLayer*, false**)

*ambaLayer = amba::amba_AT or amba::amba_LT

** Only used for CT modeling

Multi-slave socket declaration:

amba::amba_slave_socket<32, 0*> axiOUT;

* Number of channels for this socket - 0 meaning no limit

Instantiation (module constructor):

axiOUT("axiOUT", amba::amba_AXI, ambaLayer*, false**)

*ambaLayer = amba::amba_AT or amba::amba_LT

** Only used for CT modeling

Binding sockets:

Binding AXI sockets follows the rules for TLM2.0 socket binding.

Example: initiator.axi(target.axi)